Sabuni

MATERIAL SAFETY DATA SHEET

Date of issue: 3rd September 2011

Data Sheet Ref: GJ

1. Product Identification

THERROX COMFORT & THERROX 500-2000: 100 % POLYESTER. NEEDLED FELT.

2. Hazardous Ingredients

This product should NOT be considered hazardous. It is inert and non-toxic in normal handling conditions.

3. Physical Data

Physical State						Solid

Boiling Point						Not applicable

Melting Point						130 Degrees C

Vapour Pressure @ 20 Deg. C				N/A

Vapour Density						N/A

Percentage Volatiles					Less than 0, 5%

Appearance						Dark Grey

Solubility in Water					Negligible

Specific Gravity						0, 18 grammes / c.c.

Evaporation Rate					N/A

pH							N/A

Odour							None

4. Fire and Explosion Hazard Data

Auto Ignition Temperature				420 – 500 Degrees C

Extinguishing Media					Carbon Dioxide, Dry Foam

Special Fire Fighting Procedure	Standard Procedure for Class A Fires, (Use self-contained breathing apparatus)

Usual Fire & Explosion Hazards	None

Hazardous Products of Combustion	Carbon Monoxide, Carbon Dioxide

5. Health Hazard Data

Effects of Over Exposure	None are expected with normal handling. Grinding gently to avoid melting.

Threshold Limit Value	Not established

Emergency & First Aid Procedures	Seek Medical Attention if necessary

Inhalation:	Remove to fresh air

Eye Contact:	Flush with Water

Skin Contact:	Wash with Soap & Water

Ingestion:	Not applicable

Sabuni

6. Reactivity Data

Stability Conditions to Avoid	None

Incompatibility (Materials to Avoid)	Avoid contact with hot or concentrated acids,

Hazardous Polymerisation	Will not occur.

7. Spill or Leak Procedures

Steps to be taken if material is released or spilled:	If material is NOT contaminated, it can be wiped and used.

Waste Disposal Method:	Disposal must be done in accordance with relevant regulation. Sanitary landfill is recommended.

8. Special Protection Information

Appropriate Hygienic Practices	Avoid breathing fibres or dust.

Wash thoroughly after handling and before eating, drinking or smoking. Remove contaminated clothing and clean before reuse.

Personal Protective Equipment	When processing (e.g. Cutting, grinding), wear appropriate dust mask, impervious gloves, Safety Glasses and appropriate protective clothing.

Engineering Controls	Adequate ventilation should be provided during processing.

9. Labelling (CPL Regulations 1984)

Not Classified

10. Storage

Store in a dry place at a temperature between 5 deg. C. and 27 Deg. C. Store away from heat and foodstuffs.

======================================================================================

The information contained herein is furnished without warranty of any kind. Product users should make independent judgements of suitability of this information to ensure proper use and protect the Health & Safety of employees.

Before using any product, the label should be carefully read and current literature and information relating to the product consulted.