| EEC Safety Data Sheet according to Directive 91/155/EEC |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Identification of the preparation and the company |  |  |  | Revised on: January 10, 2001 |  |  |
| Product name and/or code: Foss Plast 8016 No. 2216 |  |  |  | Replaces material safety data sheet of: January 30, 1995 |  |  |
| Company: Casco A/S 2-4, Præstemosevej DK-3480 Fredensborg Phone: +45 48 46 52 00 |  | Sole distributor in UK: Fussell’s Rubber Company Limited Weston-Super-Mare, Avon Phone: (0934) 513473 |  |  |  |  |
| Type of product: Polyurethane based contact adhesive |  |  |  | Application field: Bonding PVC soles, etc. |  |  |
| 2. Composition/information on ingredients Chemical name of hazardous substances |  |  |  |  |  |  |
| Name | CAS-No. |  | Conc. range |  | Symbol | R-phrases |
| Acetone | 67-64-1 |  | 25-50% |  | F, Xi | 11-36-66-67 |
| Ethyl acetate | 141-78-6 |  | 25-50% |  | F, Xi | 11-36-66-67 |
| 3. Hazards identification |  |  |  |  |  |  |
| According to Directive 88/379/EEC the product is classified: F HIGHLY FLAMMABLE Xi IRRITANT Risk phrases: Irritating to eyes. Exposure may cause dizziness and drowsiness. |  |  |  |  |  |  |

4. First aid measures

General - Inhalation Remove to fresh air in case of accidental inhalation of vapours. Eye Contact Rinse immediately with plenty of water for at least 15 minutes. Seek medical advice. Skin Contact Remove contaminated clothes. Flush immediately with plenty of water. Ingestion Give plenty of water or milk. Do NOT induce vomiting. Seek medical advice.

5. Fire-fighting measures

Suitable extinguishing media: Foam, carbon dioxide, dry chemical or water fog. Not to be used: Water jet! Special hazards: Burning produces irritant fumes. Fire fighters should wear self-contained breathing apparatus.

6. Accidental release measures

Do not allow product to enter soil or surface water. Avoid breathing vapours. Soak up with inert absorbent material (e.g. sand, sawdust) and remove mechanically. Transport to a well-ventilated area (outside) for disposal in accordance with the waste regulations.

7. Handling and storage

Do not breathe vapour. Keep away from sources of ignition - No smoking. Keep container in a well-ventilated place. Keep away from heat. Take precautionary measures against static discharges. Keep out of reach and children.

| 8. Exposure controls/personal protection |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Local exhaust should be used to maintain levels below the TLV. For spray application or in areas without adequate ventilation, an air-supplied respirator mask should be worn. Occupational exposure limits (1994) (8 hour TWA-value) |  |  |  |  |  |  |  |  |
| Name |  | ppm | mg/m3 |  | Name |  | ppm | mg/m3 |
| Acetone |  | 750 | 1780 |  |  |  |  |  |
| Ethyl acetate |  | 400 | 1400 |  |  |  |  |  |
| Personal protection Respiratory protection: In case of insufficient ventilation wear breathing apparatus. Hand protection: Rubber gloves (butyl or nitrile rubber). Eye protection: Safety glasses. General protection and hygiene: The usual precautions for the handling of chemicals must be observed. |  |  |  |  |  |  |  |  |
| 9. Physical and chemical properties |  |  |  |  |  |  |  |  |
| Colour and form: | Yellowish liquid |  |  | Solubility in water: |  | Insoluble |  |  |
| Density: | 0.87 g/cm3 |  |  | Flash point: |  | Approx. -15°C |  |  |
| Vapour pressure (20°C): | Approx. 240 mbar |  |  | Solvent content: |  | Approx. 78% |  |  |
| Viscosity, 25°C: | Approx. 2.500 mPas |  |  | Explosion limits (vol%): |  | 2.0-13 |  |  |
| 10. Stability and reactivity |  |  |  |  |  |  |  |  |
| Stable under normal conditions. Materials to avoid: Strong bases and oxidizing agents. Hazardous decomposition products: CO, CO2, hydrochloride and NOx. |  |  |  |  |  |  |  |  |
| 11. Toxicological information |  |  |  |  |  |  |  |  |
| There is no data available on the product itself. Exposure to organic solvent vapours may result in adverse health effects such as irritation of the mucous membrane and the respiratory system and adverse effects on the renal and central nervous systems. Symptoms include headache, dizziness, fatigue, muscular weakness, drowsiness and in extreme cases, loss of consciousness. Repeated or prolonged contact with the product may lead to removal of natural fats from the skin resulting in non-allergic contact dermatitis and absorption through the skin. Splashes in the eyes may cause irritation and reversible local damage. |  |  |  |  |  |  |  |  |

| 12. Ecological information |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No information is accessible. Do not allow product to enter soil or surface water. |  |  |  |  |  |  |  |  |
| 13. Disposal considerations |  |  |  |  |  |  |  |  |
| Waste must be disposed of in accordance with local environmental control regulations. Incineration is the preferred method. EWC code: 08040200. |  |  |  |  |  |  |  |  |
| 14. Transport information |  |  |  |  |  |  |  |  |
| ADR/RID |  |  | IMDG |  |  |  |  |  |
| Class/Item number/Letter |  | TEC(R) | Class | PG | MP | UN-no. | EmS | MFAG |
| 3.5 (c) |  | 595A | 3.2 | III | - | 1133 | 3-05 | 330 |
| Proper technical name |  |  | Proper shipping name |  |  |  |  |  |
| ADHESIVES |  |  | ADHESIVES |  |  |  |  |  |
| (flash point below 23°C) |  |  |  |  |  |  |  |  |
| 15. Regulatory information |  |  |  |  |  |  |  |  |
| Labelling according to EEC Directive 88/379/EEC. |  |  |  |  |  |  |  |  |
| Danger symbol: | F, Highly flammable Xi, Irritant |  |  |  |  |  |  |  |
| Contains: | Acetone and Ethyl acetate |  |  |  |  |  |  |  |
| R-phrases: | Irritating to eyes. Exposure may cause dizziness and drowsiness. |  |  |  |  |  |  |  |
| S-phrases: | Do not breathe vapours. Keep container in a well-ventilated place. Keep away from sources of ignition - No smoking. In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. If swallowed, seek medical advice immediately and show this container or label. Use only in well-ventilated areas. Keep out of reach of children. |  |  |  |  |  |  |  |
| 16. Other information |  |  |  |  |  |  |  |  |
| The information contained in this safety data sheet is provided in accordance with the requirements of the Chemicals (Hazard Information and Packaging) Regulations. The product should not be used for purposes other than those shown in Section 1 without first referring to the supplier and obtaining written handling instructions. As the specific conditions of use of the product are outside the supplier's control, the user is responsible for ensuring that the requirements of relevant legislation are complied with. The information contained in this safety data sheet is based on the present state of knowledge and current national legislation. It provides guidance on health, safety and environmental aspects of the product and should not be construed as any guarantee of technical performance or suitability for particular applications. |  |  |  |  |  |  |  |  |