F. Ball and Co. Ltd. #### MATERIAL SAFETY DATA SHEET

| - Identification Material: Company: | Gripsotex LA60 F. Ball and Co. Ltd. Churnetside Business Park Station Road, Cheddleton Leek, Staffordshire ST13 7RS | Data Sheet Ref: 4650/4 Revision: Issue 4 Date of Issue: 19/12/02 Tel: 01538 361633 Fax: 01538 361622 |
| --- | --- | --- |

| - Composition - Substances Posing a Health Hazard: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Substance | CAS No. | Exposure Limits | Symbol | Risk Phrases | Conc. (%w/w) |
| N/A |  |  |  |  |  |
| - Substances With Recognised Exposure Limit: |  |  |  |  |  |
| Substance | CAS No. | Exposure Limits |  |  | Conc. (%w/w) |
| Ammonia | 7664-51-7 | OES 25 PPM 8Hr TWA , 35 PPM 15 min.ref. |  |  | <0.25 |
| - General Description: Aqueous dispersion of natural rubber latex with small percentages of low hazard thickener, odour modifier and trace stabilising agents including non-metallic biocide. Natural Rubber latex contains small quantities of ammonia as a stabilising agent. |  |  |  |  |  |

- Hazards Identification Most Important Hazards to Man: Prolonged or repeated contact with the skin may cause dermatitic effects in sensitive individuals. Most Important Hazards to the Environment: Product is miscible with water and will discolour the water. Discolouration will be evident far downstream.

- First Aid Measures ##### Inhalation: Move affected person to fresh air. If symptoms persist, seek medical advice. ##### Skin Contact: Wash affected areas with soap and water. If irritation persists, seek medical advice. ##### Eye Contact: Irrigate the eye immediately with plenty of water or suitable proprietary eyewash for at least fifteen minutes. If irritation persists, seek medical advice. ##### Ingestion: Wash mouth out with water but do not swallow. Give plenty of water to drink. Do not induce vomiting. Obtain immediate medical advice.

- Fire Fighting Measures Product is not flammable as supplied. Dried film will burn in a general fire situation to release oxides of carbon. Use extinguishing media suitable for the general fire situation. Fire fighters must wear self-contained breathing-air apparatus.

- Accidental Release Measures Small spillages should be absorbed onto sand or similar inert inorganic material and transferred to a suitable labelled container prior to disposal. Larger spillages should be retained by a sand barrier and collected into a suitable labelled container prior to disposal. Prevent product from entering drains or other watercourses. Wear suitable protective equipment to prevent skin or eye contact. Avoid breathing product vapours.

- Handling and Storage Store product away from heat sources and foodstuffs in tight sealed containers. Store away from direct sunlight at a temperature between 5ºC and 30ºC in dry well-ventilated areas. Protect from frost. Avoid transfer of product from hand or glove to face, eyes or lips. Employ good hygiene practices, wash hands before meal or beverage breaks and at the end of work periods. Smoking and the preparation or consumption of food or drink in the work area should be prohibited. Avoid frequent or prolonged contact with the skin.

- Exposure Controls/Personal Protection Ensure provision of constant fresh air ventilation during and immediately after use. Use mechanical means where natural ventilation facilities are inadequate. Maintain ammonia in-air levels below the occupational exposure limit: Wear eye protection against splash hazard. Use impervious gloves where possibility of skin contact may occur.

- Physical and Chemical Properties Appearance: White aqueous emulsion. Odour: Ammoniacal. pH: 9.5 - 10.5 Volatiles: 40% approx. Rel Density: 0.90 @ 20ºC approx. Flashpoint: N/A Viscosity: 50 - 150 centiPoise @ 20ºC Other: N/A

- Stability and Reactivity Stable under normal conditions of use. Incompatible with acids and oxidising agents.

- Toxicological Information Inhalation: Exposure to levels of ammonia slightly in excess of the occupational exposure limits may cause some irritation of the nose. Higher concentrations of vapour can cause irritation of the respiratory tract, difficulty with breathing and lachrymatory effects. Skin Contact: Frequent or prolonged contact may cause reddening, dryness and soreness of the skin. Eye Contact: Causes reddening and soreness of the eyes. Ingestion: Product coagulates in the stomach and may cause gastric discomfort and pain.

- Ecological Information Product should not be allowed to enter natural waters without pre-treatment.

- Disposal Considerations Small quantities may be mixed with inert material, allowed to dry and disposed of as low hazard general solid waste. Large quantities should be disposed of via licensed waste operators as required under local and national regulations.

| - Transport Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| UN Number: - | Proper Shipping Name: - |  | Class: - | Packing Group: - |
| RID/ADR class: - | IMDG Class: - | Marine Pollutant: - |  |  |

| - Regulatory Information Classification / Symbol: Contains: N/A |  |  |
| --- | --- | --- |
| Risk Phrases: |  |  |
| N/A |  |  |
| Safety Phrases: |  |  |
| N/A |  |  |
| The information contained in this data sheet does not constitute the user's own assessment of workplace risk as required by other health and safety legislation. |  |  |

- Other Information EH40 (amended annually).