F. Ball and Co. Ltd. #### MATERIAL SAFETY DATA SHEET

| - Identification Material: Company: | Gripsotite S10 Primer F. Ball and Co. Ltd Churnetside Business Park, Station Road, Cheddleton, Leek, Staffordshire. ST13 7RS | Data Sheet Ref: 3700/5 Revision: Issue 5 Date of Issue: 19/12/02 Tel: 01538 361633 Fax: 01538 361622 |
| --- | --- | --- |

| - Composition - Substances Posing a Health Hazard: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Substance | CAS No. | Exposure Limits | Symbol | Risk Phrases | Conc. (%w/w) |
| Ethyl Acetate | 141-78-6 | OES 200 PPM 8 Hr TWA 400 PPM 15 min. ref. | F, Xi | R11, R36, R66, R67 | 40 - 50 |
| Butanone | 78-93-3 | OES 200 PPM 8 Hr TWA 300 PPM 15 min. ref. | F, Xi | R11, R36, R66, R67 | 40 - 50 |
| - Substances With Recognised Exposure Limit: |  |  |  |  |  |
| Substance | CAS No. | Exposure Limits |  |  | Conc. (%w/w) |
| - | - | - |  |  | - |
| - General Description: Solution of resins and elastomers in a blend of organic solvents. |  |  |  |  |  |

- Hazards Identification Most Important Hazards to Man: Vapour is irritant, may cause drowsiness, nausea, dizziness and other narcotic effects. Liquid acts as a skin degreaser and may with frequent or prolonged contact, cause dermatitis. Ingested liquid will cause irritation of the digestive tract and may cause involuntary vomiting which may then be inhaled. Liquid and its vapours are irritating to the eyes. Most Important Hazards to the Environment: Solvent contamination of drains, surface waters and land. VOC emissions to air. Potential fire and explosion hazard.

- First Aid Measures ##### Inhalation: Remove affected person to fresh air. Keep warm and at rest. In the event of severe effects of exposure, obtain immediate medical aid and apply appropriate first-aid techniques. ##### Skin Contact: Wash affected area thoroughly with soap and water. If irritation persists, seek medical advice. ##### Eye Contact: Irrigate the eyes immediately with water or suitable proprietary eyewash for at least fifteen minutes. Ensure product is flushed from under the eyelids. Obtain medical attention. ##### Ingestion: Wash the mouth out with water, but do not swallow. Drink one or two pints of water. Do not induce vomiting. Obtain immediate medical attention.

- Fire Fighting Measures Extinguish with foam, CO2 or dry powder appliances. Do not use water jets. Cool containers adjacent to the fire area with water spray. Fire fighters must wear self-contained breathing-air apparatus.

- Accidental Release Measures Extinguish any naked flames or other sources of ignition. Evacuate non-essential personnel from the area. Absorb onto sand or other inert, inorganic material and place into suitable sealed, labelled containers prior to disposal. Prevent material from entering drains or other watercourses. Avoid inhaling vapours, wear suitable protective equipment to avoid skin or eye contact.

- Handling and Storage Store in a well ventilated place at a temperature between 5°C and 30°C under conditions that comply with the Highly Flammable Liquids and Liquefied Petroleum Gases Regulations 1972. Store away from heat sources, any means of ignition and out of direct sunlight. Store away from foodstuffs and oxidising agents. Protect containers from mechanical damage. A strict no-smoking rule must be enforced. Notices to prohibit smoking and the use of naked lights should be posted. Keep containers tight sealed when not in use and avoid breathing product vapours. Suitable fire extinguishers should be kept on hand. Free access to safe exits must be maintained. Maintain Acetone in-air levels below the Occupational Exposure Limits. Exercise particular care in low-lying or confined spaces to prevent creation of hazardous vapour/air mixtures. Avoid transfer of product from hand or glove to face, eyes or lips. Prohibit smoking and the use of naked lights in or around the work area. Do not smoke or approach a means of ignition if product contaminates clothing. Remove contaminated clothing and launder before re-use. Employ good hygiene practices, wash hands before food or beverage breaks and at the end of work periods. The preparation or consumption of food or drink in the work area should be prohibited.

- Exposure Controls/Personal Protection Ensure constant supply of fresh air ventilation during and immediately after use. Use flame proof means where natural ventilation facilities are inadequate and / or use suitable RPE such as organic vapours filter or air supply. Goggles or faceshield must be used where danger of splash or eye contact exists. Use suitable barrier cream or impervious gloves where risk of skin contact may occur.

- Physical and Chemical Properties Appearance: Clear mobile liquid. Odour: Fruity odour. pH: N/A Volatiles: 87% approx. Rel Density: 0.9 @ 20°C approx. Flashpoint: -6°C approx. Viscosity: N/A. Other: N/A.

- Stability and Reactivity Stable under normal conditions of use. Incompatible with strong acids and oxidising agents.

- Toxicological Information Inhalation: Prolonged exposure to concentrations of product vapours slightly in excess of Occupational Exposure Limits may cause dizziness, nausea, light-headedness and irritation of the respiratory tract. Higher concentrations of vapour cause drowsiness, headache, mental confusion and other narcotic and anaesthetic effects. Skin Contact: Product has a de-fatting effect on skin and may cause the skin to crack, with risk of secondary infection and dermatitis. Eye Contact: Causes irritation and reddening of the eye. Causes corneal burns on prolonged contact or failure to irrigate the eye adequately. Ingestion: Causes irritation of the throat and alimentary tract, with possibility of nausea and involuntary vomiting. Vapours from the stomach may be inhaled and create similar hazards as for inhalation.

- Ecological Information Product must not be released to natural waters without pre-treatment.

- Disposal Considerations Dispose of via licensed waste operators, ultimately to an approved incineration unit for organic solvents.

| - Transport Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| UN Number: 1133 | Proper Shipping Name: Adhesives |  | Class: 3 | Packing Group: II |
| RID/ADR class: 3 | IMDG Class: 3 | Marine Pollutant: No |  |  |

| - Regulatory Information Classification / Symbol: Contains: - |  |  |
| --- | --- | --- |
| Risk Phrases: |  |  |
| R11 | Highly Flammable. |  |
| R36 | Irritating to eyes. |  |
| R66 | Repeated exposure may cause skin dryness or cracking. |  |
| R67 | Vapours may cause drowsiness or dizziness. |  |
| Safety Phrases: |  |  |
| S9 |  | Keep container in a well ventilated place. |
| S16 |  | Keep away from sources of ignition - No Smoking. |
| S25 |  | Avoid contact with eyes. |
| S26 |  | In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. |
| S33 |  | Take precautionary measures against static discharges. |
| S37 |  | Wear suitable gloves. |
| S51 |  | Use only in well ventilated areas. |
| S60 |  | This material and its container must be disposed of as hazardous waste. |
| The information contained in this data sheet does not constitute the user's own assessment of workplace risk as required by other health and safety legislation. |  |  |

| - Other Information EH40 (amended annually) The Highly Flammable Liquids and Liquefied Petroleum Gases Regulations 1972 |  |
| --- | --- |

The information contained in this data sheet is to the best of F. Ball and Co. Ltd's knowledge correct as at the date of publication. The information relates only to product manufactured after the issue date of this sheet. If the product you are assessing was manufactured prior to this issue date, please contact F. Ball and Co. Ltd. QES Department for further advice.