F. Ball and Co. Ltd. #### MATERIAL SAFETY DATA SHEET

| - Identification Material: Company: | Astral Yellow Beeswax F. Ball and Co. Ltd Churnetside Business Park, Station Road, Cheddleton, Leek, Staffordshire. ST13 7RS | Data Sheet Ref: AYBW/4 Revision: Issue 4 Date of Issue: 12/12/02 Tel: 01538 361633 Fax: 01538 361622 |
| --- | --- | --- |

| - Composition - Substances Posing a Health Hazard: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Substance | CAS No. | Exposure Limits | Symbol | Risk Phrases | Conc. (%w/w) |
| - | - | - | - | - | - |
| - Substances With Recognised Exposure Limit: |  |  |  |  |  |
| Substance | CAS No. | Exposure Limits |  |  | Conc. (%w/w) |
| - | - | - |  |  | - |
| - General Description: Wax of animal origin. |  |  |  |  |  |

- Hazards Identification Most Important Hazards to Man: Solid product: - None identified. Molten product: - Heated product may give of irritating vapours. Molten product may splash and cause burns. Most Important Hazards to the Environment: Solid product: - None identified. Molten product: - Product will solidify in contact with water blocking watercourses.

- First Aid Measures ##### Inhalation: Solid product: - N/A Molten product: - Remove affected person to fresh air. If symptoms persist, seek medical advice. ##### Skin Contact: Solid product: - Wash with soap and water. Molten product: - Flush immediately with cold water. Do not attempt to remove solidified product from the skin. Obtain immediate medical attention. If solidified material encircles a limb or the body, the adhering wax should be split to avoid tourniquet effect. ##### Eye Contact: Solid product: - Irrigate the eye immediately with water or suitable proprietary eyewash for at least fifteen minutes. If symptoms persist, seek medical advice. Molten product: - Flush immediately with plenty of water. Do not attempt to remove solidified product. Obtain immediate medical attention. ##### Ingestion: Solid product: - Rinse mouth with plenty of water. If symptoms persist, seek medical attention. Molten product: - Ensure airways are clear. Obtain immediate medical attention.

- Fire Fighting Measures Product will burn in a general fire situation to produce noxious oxides of carbon and thick black smoke. Extinguish with foam, CO2 or dry powder appliances. Do not use water jets. Fire fighters must wear self-contained breathing-air apparatus.

- Accidental Release Measures Solid product should be swept or picked up as appropriate. Molten material should be retained by sand or similar inert material and allowed to cool to ambient temperatures. Once cool, material may be picked or swept up and placed into a suitable container prior to disposal.

- Handling and Storage Store in a dry place at a temperature between 5°C and 30°C. Store away from heat sources. Smoking and the use of naked lights in the workplace should be prohibited. Product is a low hazard solid at ambient temperatures, but may create vapour and burns hazards from use of molten product. Avoid transfer of product from hand or glove to face, eyes or lips. Employ good hygiene practices, wash hands before meal or beverage breaks and at the end of work periods.

- Exposure Controls/Personal Protection Product is a low hazard material at ambient temperatures. Ensure good ventilation if using molten material. Vent wax vapours to a safe location. Suitable protective clothing and eye/face protection should be employed if molten product is used.

- Physical and Chemical Properties Appearance: Dark yellow waxy solid. Odour: Slightly sweet. pH: N/A. Volatiles: N/A. Rel Density: Not established. Flashpoint: N/A. Viscosity: N/A. Other: N/A

- Stability and Reactivity Stable in normal use. Incompatible with powerful oxidising agents. Overheating will result in emission of noxious fumes.

- Toxicological Information Inhalation: Solid product: - None identified. Molten product: - Fumes from molten product may be harmful. Skin Contact: Solid product: - None identified. Molten product: - Molten material will adhere to and burn the skin. Eye Contact: Solid product: - Transient mechanical irritation of the eye. Molten product: - Molten material will adhere to and burn the eye. Ingestion: Solid product: - May cause gastric disomfort. Molten product: - Product will adhere to and burn the gastro-intestinal tract.

- Ecological Information Product is not readily biodegradable. Molten product will solidify on contact with water to block drains and other watercourses.

- Disposal Considerations Product should be disposed of as low hazard solid waste.

| - Transport Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| UN Number: - | Proper Shipping Name: - |  | Class: - | Packing Group: - |
| RID/ADR class: - | IMDG Class: - | Marine Pollutant: No |  |  |

| - Regulatory Information Classification / Symbol: Contains: - |  |  |
| --- | --- | --- |
| Risk Phrases: |  |  |
| - |  |  |
| Safety Phrases: |  |  |
| - |  |  |
| The information contained in this data sheet does not constitute the user's own assessment of workplace risk as required by other health and safety legislation. |  |  |

| - Other Information |  |
| --- | --- |

The information contained in this data sheet is to the best of F. Ball and Co. Ltd's knowledge correct as at the date of publication. The information relates only to product manufactured after the issue date of this sheet. If the product you are assessing was manufactured prior to this issue date, please contact F. Ball and Co. Ltd. QES Department for further advice.